SEB_

QVI

Description &

FPGA/CPLD

Specification

(Rev. 1.2)

Revision History

| Revision | Date | Author | Comments |
| --- | --- | --- | --- |
| 1.0 | Feb. 22, 2005 | Doug Wilson | Initial Draft Release |
| 1.1 | Sep 14,2005 | Dave Cheung | Initial Alpha Release |
| 1.2 |  | Dave Cheung | Updated: Changes should be in Blue ink. + Expanded Overview + Added Functional Description section + Packed Interrupt Register to combine Status / En + Added Missing Host DMA Address Reg + Added Missing Device Memory Address Reg + Modified Power Amp Rail Voltage Reg + Added Missing Kelvin Test relay bits to DUT Connect Reg + Added Range Selected CAL Registers for Force - Removed CAL registers for Measure. To be performed by Host PC + Added Memory Type select register for BAR1 address access |

Revision History	2

1.	Overview	5

1.1	Functional Description	7

1.2	Additional Features	8

1.3	Controls	11

1.3.1	FPGA to PCI bus interface signals	11

1.3.2	FPGA to U11 interface signals	11

1.3.3	FPGA to CP1_U1 interface signals	11

1.3.4	FPGA to CP1_U2 interface signals	11

1.3.5	FPGA to VMU1 interface signals	11

1.3.6	FPGA to VMU2 interface signals	12

1.3.7	FPGA to VMU3 interface signals	12

1.3.8	FPGA to VMU4 interface signals	12

1.3.9	FPGA to VP Power CH1, CH2	12

1.3.10	FPGA to VP Power CH3, CH4	12

2.	SEB VI FPGA and CPLD	13

2.1	SEB QVI FPGA and Host CPU Interaction	13

2.2	Registers	14

2.2.1	PCI Config Registers	14

2.2.1.1	PCI Command Register	14

2.2.1.2	PCI Status Register	15

2.2.1.3	PCI BASEADR0 (Base Address 0) Register	16

2.2.1.4	PCI BASEADR1 (Base Address 1) Register	16

2.2.1.5	PCI INTPINSEL (Interrupt Pin Select) Register	16

2.2.2	Memory Mapped Registers	17

2.2.2.1	SLOTID  Register	19

2.2.2.2	Interrupt Enable / Status Register	19

2.2.2.3	Timer Counter Control / Status Register	20

2.2.2.4	DMA Transfer Length Register	20

2.2.2.5	Host DMA Buffer Address Register	20

2.2.2.6	Device Memory Address Register	20

2.2.2.7	DMA Control Register	21

2.2.2.8	Channel_Select Register	22

2.2.2.9	PowerAmp Rail Voltage Register	23

2.2.2.10	Voltage_Force Register	24

2.2.2.11	Current_Force Register	26

2.2.2.12	DUT_Connect Register	28

2.2.2.13	VMU_Input Register	30

2.2.2.14	VDAC_A Register	32

2.2.2.15	VDAC_B Register	32

2.2.2.16	IDAC_A Register	33

2.2.2.17	IDAC_B Register	33

2.2.2.18	Force Trigger Delay Register	34

2.2.2.19	Pulse Width Register	34

2.2.2.20	ADC_Measure Register	35

2.2.2.21	Measure Trigger Delay Register	36

2.2.2.22	Sampling Period Register	36

2.2.2.23	Samples Register	36

2.2.2.24	Start Trigger Register	37

2.2.2.25	ADC Accumulator Register	38

2.2.2.26	ADC Digitizing Memory	38

2.2.2.27	Board Info EPROM Memory	39

2.2.2.28	Power Status Register	39

2.2.2.29	Voltage DAC Gain Register	40

2.2.2.30	Voltage DAC Offset Register	40

2.2.2.31	Current DAC Gain Register	40

2.2.2.32	Current DAC Offset Register	40

2.2.2.33	BAR1 Memory Type Register	41

2.2.2.34	Image Update Register	41

3.	HARDWARE DESIGN INFO	42

3.1	FPGA – U17	42

3.1.1	FPGA Pin Map	42

3.1.2	FPGA Pin Description	47

3.1.3	FPGA (U17) UCF	49

3.2	CPLD U11	54

3.2.1	CPLD Pin Map	54

3.2.2	CPLD Pin Description	55

3.2.3	U11 UCF	56

3.3	CPLD CPx_U1	58

3.3.1	CPLD PIN Map	58

3.3.2	CPLD PIN Description	59

3.3.3	CPx_U1 UCF	60

Figure 1.	VI Configurations Block Diagram	5

Figure 2.	VI Functional Block Diagram	8

Figure 3.	KELVIN High Continuity Test	8

Figure 4.	KELVIN LOW Continuity Test	9

Figure 5.	Pulsing VDAC or IDAC	9

Figure 6.	ADC sampling events	10

Figure 7.	QVI’s Programmable Components	11

Figure 8.	FPGA Functional Blocks	13

- Overview

The SEB QVI (Quad VI) has a total of 4 channels, 2 channels (CH1, CH2) capable of 5A each and 2 channels (CH3, CH4) capable of 1A max. All four channels are floating with respect to the digital interface and chassis ground.  All four VI’s are stackable to produce twice the voltage.  Only channels 1,2 are “gang-able” to produce twice the current. See figure below for Stacking and Ganging DUT board connections.

See figure below for Stacking and Ganging DUT board connections.

- VI Block Diagram of Various Configurations Block Diagram

- Two channels (1,2 or 3,4) of VI share the Rail supplies as shown above, to conserve PCB space.  The available configurations and their setups are listed in Table 1 below.

- When the DUT board connects to the VI as shown in the “Norm box” (above figure), 2 channels of VIs are available for use.  The Low Side of the VI should be connected to DUT GND and the High Side be connected to the DUT Pins.  Table 1 rows 1-7 show the various Voltage Swings available and their respective setups of VPM, VPP DACs and the relays.  This is different from the Voltage Range Selection that controls the Gain Stage of the Output Buffer.

- In the Gang configuration, the 2 VI channels (high side) are connected to a single DUT pin.  The Rails for the Power Amp has to be the same, hence the Relays has to be selected together.  The Vfollow relay has to be close in order to make the second channel follow the first.  This guarantees that the two channels track each other.

- In the Stack configuration, one channel’s high side is connected to the DUT ground and the other channel’s high side is connected to a DUT pin.  Which is which depends on whether a positive or a negative voltage is required.

- Configurations

The following are the configuration permutations of the VI and their respective Rail parameter DAC and relay settings.  Don’t panic, some of these parameters will be automatically set by the FPGA/CPLD.

|  | Swing | CH1 | CH2 | VPP | VPM | +KVPPP1 | K-VPM1 | K+VPP2 | K-VPM2 | KVfollow | DUT Bd |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10V Swing | ±1V, ±5V | ±1V, ±5V | x | x | Open | Open | Open | Open | Open | Norm |
| 2 | 50V Swing | ±25V | ±25V | +30V | -30V | Close | Close | Close | Close | Open | Norm |
| 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ±25V | -5V to +25V | +30V | -30V | Close | Close | OpenClose | CloseOpen | Open | Norm |
| 4 |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | -5V to +25V | ±25V | +30V | -30V | OpenClose | CloseOpen | Close | Close | Open | Norm |
| 6 |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ±25V | +5V to –25V | +30V | -30V | Close | Close | Open | Close | Open | Norm |
| 8 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | +5V to –25V | ±25V | +30V | -30V | Open | Close | Close | Close | Open | Norm |
| 9 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | -5V to +50V | -5V to +50V | +55V | x | Close | Open | Close | Open | Open | Norm |
| 10 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | +5V to –50V | -5V to +50V | +55V+55V | -55V-55V | Open | Close | Close | Open | Open | Norm |
|  |  | -5V to +50V | +5V to –50V | +55V | -55V | Close | Open | Open | Close | Open | Norm |
|  |  | +5V to –50V | +5V to –50V | x | -55V | Open | Close | Open | Close | Open | Norm |
| 11 | 10V, 10A* | ±1V, ±5V | Follow CH1 | x | x | Open | Open | Open | Open | Close | Gang |
| 12 | 50V, 10A*50V, 10A* | ±25VAny of above | Follow CH1 | +30V+55V | -30V-55V | CloseAbove | CloseAbove | Close+VP1 | Close-VP1 | Close | Gang |
| 13 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | -5V to +50V | Follow CH1 | +55V | x | Close | Open | Close | Open | Close | Gang |
| 14 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | +5V to –50V | Follow CH1 | x | -55V | Open | Close | Open | Close | Close | Gang |
| 15 | 100V Swing | -5V to 100V | DUT GND | +55V | -55V | Close | Open | Open | Close | Open | Stack |
| 16 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DUT GND | -100V to +5V | +55V | -55V | Close | Open | Open | Close | Open | Stack |

- Configurations Table

Note:  	* = Only available for Channels 1 and 2.  Channel 3 and 4 are 1A max only.

VPP= VP + or Positive Rail Voltage, VPM= VP – or Negative Rail Voltage, specified through PowerAmp Rail Voltage Register.

KVPXX = Relays to Connect the VPP and VPM supplies to the Rails of the VI Power Amp.  These relays are automatically setup via Voltage_Force Register

It is always a good practice to program VPP or VPM with just enough headroom for the maximum swing to conserve power dissipation.  Remember Power dissipated = (VP – Vout) * Current.

Example1:  Need 50V swing, 0 to +50V on channel1 and 0 to –50V on channel2.  Here’s what to do: (Entry # 9)

- Program CH1_VP+ to +55V (5V headroom) by closing KVPP1 and program VPP DAC = +55V.

- Program CH1_VP- to –12V by opening KVPM1, it will go to –12V.

- Program CH2_VP+ to +12V by opening KVPP2, it will go to +12V.

- Program CH2_VP- to –55V by closing KVPM2 and program VPM DAC = -55V.

- Connect DUT ground to VI common and common_sense and the 2 channels’ Force and Sense to DUT Pins.

- Program the Appropriate Voltage Range for each channel.  Refer to Voltage_Force register.

Example2:  Need 100V swing, 0 to –100V.  Here’s what to do: (Entry #16)

- Program CH1_VP+ to +55V by closing KVPP1 and program VPP DAC = +55V.

- Program CH1_VP- to –12V by opening KVPM1,  it will go to –12V.

- Program CH2_VP+ to +12V by opening KVPP2,  it will go to +12V.

- Program CH2_VP- to –55V by closing KVPM1 and program VPM DAC = -55V.

- Connect DUT ground to CH1 Force & Sense and CH2 Force & Sense to DUT pin that needs –100V.

- Connect at DUT board, Common and Common_Sense of VI pair.

- Program the Appropriate Voltage Range for each channel.  Refer to Voltage_Force register.

- Example3:  Need 50V swing, 0 to +50V at 10Amps.  Here’s what to do: (Entry #13)

- Program VPP DAC = +55V,  (VPM DAC = don’t care).

- Open KVPM1, and KVPM2…. This will set CH1_VP- and CH2_VP- to –12V.

- Close KVPP1 and KVPP2 … This will set CH1_VP+ and CH2_VP+ to VPP DAC or +55V

- Close KVfollow relay

- Connect DUT board Ground to Common, and connect CH1 AND CH2 Force and Sense to DUT pin.

- Program the Appropriate Voltage Range for each channel.  Refer to Voltage_Force register.

  - Functional Description

  - The following figure (Fig 2) represents the functional block diagram of a single channel VI.  As mentioned before, channels 1 and 2 can deliver up to 5A each while channels 3 and 4 can deliver up to 1A each.  Channels 3 and 4 cannot be ganged to produce 2A.

  - Conventions: The colored boxes with underline, bold text represents programmable functions.  Underlined and capitalized text represents the relay names.  Capitalized and Bold text on the right represents the signals that connect to the DUT board.

  - The VI is floating until the user connects the FORCE and COMMON to the DUT GROUND and DUT PIN.  Figure 1 shows the 3 ways to connect the VI to the DUT.  This VI provides 4 wire connections (FORCE, SENSE, COMMON, COMMON_SENSE).   If a 2 wire connection is desired, simply program the DUT_Connect Register to enable local sense.

  - The VI has 2 control loops, voltage and current.  Which takes control depends on the load (DUT).   To set the voltage, one must program the range (VRng), the value (VDac) and/or the compensation (VComp), which is optional, all in the Voltage_Force Register.  To set the current, the range (IRng), the value (IDac) and optional compensation (IComp) can be programmed through the Current_Force Register.

  - The VI has a feature which allows the user to control current loop external to the VI.  The programming is done through the DUT_Connect Register’s external loop control.  A single bit in the DUT_Connect Register (Output Relay) controls all 5 signals (FORCE, SENSE, GUARD, COMMON, COMMON_SENSE).

  - The KELVIN_HI and KELVIN_LO circuit offers the user a means to measure the sense connections (High and low sense). This will be described in the additional feature section.

- VI Functional Block Diagram

- For the measure side, each VI has its own ADC.  The measurement circuits are referred to as VMU which stands for Voltage Measurement Unit.  The VMU can measure the Voltage, Current, High Sense Test circuit, Low Sense Test circuit and a separate DUT channel.  The selection is made through the VMU_Input Register.

- The Voltage measure is between Vmeasure point (see fig 2) and the common_sense which is effectively the high sense and the low sense.  For Current measure, the VMU is measuring between Imeasure point (Isense) and the common.  For the DUT, it’s between DUT+ and DUT- pins.  For high sense Kelvin test, it is between the KELVIN+ pin from the DUT board and the High Sense, which is the Vmeasure point.  For the low sense Kelvin test, it is between the common_sense and the COM_KELVIN pin from the DUT board.  The Kelvin test features will be explained in the next section.

- The VMU range can be selected through programming the VMU_Input Register.  The ADC can be sampled through the ADC_Measure Register.  This register contains the hardware alarms which can be used to determine whether the ADC measurement is valid.

  - Additional Features

The SEB QVI has the following additional features:

- KELVIN connections continuity tests:

2 Relays can be added on the DUT board to connect the KELVIN+ pin to the DUT pin that is connected to the FORCE pin of the VI and the COMMON_KELVIN pin to DUT Pin that is connected to the COMMON pin of the VI.

To test the high sense connection, figure 3 shows the equivalent circuit.  The Kelvin_Hi relay must be closed, and a small current is to be forced and the voltage measurement is taken across KELVIN+ and Vmeasure (or Sense).

- KELVIN High Continuity Test

To test the Low Sense connection, Figure 4 shows the equivalent circuit.  The Kelvin_Lo relay is to be closed and a current to be force and the voltage measurement is to be made across the Common_Sense and COMMON_KELVIN point.

- KELVIN LOW Continuity Test

- ALTERNATE FORCING:

When writing to either Voltage_Force Register or Current_Force Register, the programmed value is clocked into the DAC immediately.  In some cases, a more precise timing control is desired to orchestrate the various VI’s.  A feature is available on this VI for the user to specify when the DAC is to be applied with the desired value.  This feature is shared among all the 4 channels within the QVI, i.e. all channels’ DAC will be updated at the end of the delay and Width point.

Figure 5 below shows the general transitions.

- Pulsing VDAC or IDAC

- To use this feature:

- First the trigger point must be defined.  The possible triggers are defined in Force Trigger Source Select field of Start Trigger Register.

- Then a delay (which can be 0) has to be specified which can be done via Force Trigger Delay Register

- The first sets of Programmed Values (PV1) are required for VDAC_A Register and IDAC_A Register of each channel.  To avoid transition, the PV1 must be the same as the initial value.

- A duration (Width, which can be 0) has to be specified via Pulse Width Register

- The second sets of Programmed Values (PV2) have to be specified for VDAC_B Register and IDAC_B Register for each channel.

- If the completion of these events is to be communicated back to the Host, the interrupt has to be enabled at Interrupt Enable / Status Register.

- Once everything is setup, including the measurement side, writing to  Start Trigger Register will kick off the chain of events.

- ALTERNATE MEASURE:

- When accessing the ADC_Measure Register, the ADC is sampled once and converted “immediately” (relative). If multiple measurements are desired then figure 6 shows what can be done with this VI.

- ADC sampling events

- To use this feature :

- First the trigger point must be defined.  The possible triggers are defined in Measure Trigger Source field of Start Trigger Register.

- Then a delay (which can be 0) has to be specified which can be done via Measure Trigger Delay Register

- A repetitive duration (Period) has to be specified via Sampling Period Register.

- The number of samples (minimum=1) has to be specified via Samples Register.

- If the completion of these events is to be communicated back to the Host, the interrupt has to be enabled at Interrupt Enable / Status Register.

- Once everything is setup, including the forcing side, writing to  Start Trigger Register will kick off the chain of events.

The ADC values go to two places;

- Up to 1K depth memory (ADC Digitizing Memory) which is per channel each.

- 2’s complement Signed Accumulator (ADC Accumulator Register) which accumulates all the measurements.  To get the average, the Host must divide the reading from ADC Accumulator Register by the number stored in Samples Register.  Then the offset and gain should be applied to obtain the calibrated average result.

- Each ADC can measure one of the following:

  - Respective Channel’s Voltage

  - Respective Channel’s Current

  - DUT Board Channel

  - Respective Channel’s High Sense Contact Test Point

  - Respective Channel’s Low Sense Contact Test Point

- Programmable trigger source for timeouts;  (for both ADC and DAC)

  - Register Write

  - Trigger Bus

- Digitizer: User can program the following parameters when using the ADC:

  - Trigger source

  - Delay after trigger (0~20ms @ 100ns resolution)

  - Period between samples (0-20ms @ 100ns resolution, to cover 50Hz)

  - Number of samples (up to 1024)

All the above values will be stored in a 1k deep SRAM inside FPGA.

- Accumulator (2’s complement signed) for averaging.  
The accumulator will be loaded at the time the digitizing memory is loaded.  The accumulator will accumulate signed (2’s complement) values.  
To compute the average, all SW has to do is to read the accumulated value and divide by the number of samples user specified.  Then the offset and gain should be applied to obtain the averaged, calibrated value of the measurement.

- Pulsed function which user can specify time delay time after trigger and pulse width (duration).  This feature will set the DAC to the first voltage after specified delay from the trigger point and then set the DAC to the second voltage after the pulse width time has been expired.  The timing resolution is 100ns.

  - Controls

The FPGA is referenced at Digital Ground.  Two CPLD, 2 channels per, are floated along with the VI channels.  The FPGA communicates with the CPLD through a serial bus, and opto-couplers.  A third CPLD is at the same level as the FPGA interfaces with the 4 VMUs (Voltage Measurement Units) through opto-couplers also.    The Supplies for the Power Amp is programmable (for minimizing power consumption) are controlled by the FPGA directly (via opto-couplers).

The following diagram shows the 3 programmable parts, within the QVI board, interfacing with FPGA.

- QVI’s Programmable Components

    - FPGA to PCI bus interface signals

- AD (32), CBE (4), FRAME_L, IRDY_L, TRDY_L, DEVSEL_L, STOP_L, PERR_L, SERR_L,  RST_L,  INTB_N, REQ_N, GNT_N, IDSEL.

    - FPGA to U11 interface signals

- CPLD_SDATAIN, CPLD_SCLK, CPLD_STRB, CPLD_SDATAOUT

    - FPGA to CP1_U1 interface signals

- B12_SCLK, B12_SDATA_IN, B12_SDATA_OUT, B12_STRB1, B12_STRB2

    - FPGA to CP1_U2 interface signals

- C34_SCLK, C34_SDATA_IN, C34_SDATA_OUT, C34_STRB1, C34_STRB2

    - FPGA to VMU1 interface signals

- ADC_CLK1, ADC_BUSY1, ADC_CNVST1, ADC_SDATA1, IN_OVRNG1, OUT_OVRNG1

    - FPGA to VMU2 interface signals

- ADC_CLK2, ADC_BUSY2, ADC_CNVST2, ADC_SDATA2, IN_OVRNG2, OUT_OVRNG2

    - FPGA to VMU3 interface signals

- ADC_CLK3, ADC_BUSY3, ADC_CNVST3, ADC_SDATA3, IN_OVRNG3, OUT_OVRNG3

    - FPGA to VMU4 interface signals

- ADC_CLK4, ADC_BUSY4, ADC_CNVST4, ADC_SDATA4, IN_OVRNG4, OUT_OVRNG4

    - FPGA to VP Power CH1, CH2

- DC1_8BIT_SYNCA, DC1_8BIT_SYNCB, DC1_MVHP_ONN, DC1_PVHP_ONN,

- DC2_8BIT_SYNCA, DC2_8BIT_SYNCB, DC2_MVHP_ONN, DC2_PVHP_ONN,

- DC12_8BIT_SCLK, DC12_8BIT_SDIN

    - FPGA to VP Power CH3, CH4

- DB3_8BIT_SYNCA, DB3_8BIT_SYNCB, DB3_MVHP_ONN, DB3_PVHP_ONN,

- DB4_8BIT_SYNCA, DB4_8BIT_SYNCB, DB4_MVHP_ONN, DB4_PVHP_ONN,

- DB34_8BIT_SCLK, DB34_8BIT_SDIN

- SEB VI FPGA and CPLD

The SEB VI FPGA provides the host interface and control logic for operating the VI.  The SEB_VI FPGA resides on the SEB_DIGITAL board and interface with the VI board via a 2mm hard metric 110pin connector.  It is designated as U17 on the SEB_DIGITAL board. The SEB VI FPGA is implemented in a Xilinx XC4VLX25 Virtex4 FPGA using a serial EEPROM that is loaded at system power-up and is provided in a 668-pin BGA package.

  - SEB QVI FPGA and Host CPU Interaction

The host system (PC) and the SEB QVI FPGA communicate through the use of memory mapped PCI register accesses and the PCI interrupt mechanism via Stargen Fabric and Bridge chipset.  The SEB QVI FPGA appears as a memory mapped target to the host system.  All VI functional operations are performed by host CPU read/write memory accesses to the SEB QVI FPGA.  PCI interrupts are mask-able for VI operation completion interrupts.

The  SEB QVI FPGA utilizes a 32-bit/33MHz PCI core from Xilinx.  This core provides the direct interface to the PCI bus, and contains user interface signals for connecting application specific logic, such as the VI functions.

The Initiator Engine connects to the user interface of the PCI core and provides the block mode DMA function.  This module contains two FIFOs. One FIFO is for data transfer from the PCI bus to the local on-chip buffers, and is referred to as the initiator read FIFO. The other FIFO is for data transfer in the opposite direction, and is referred to as the initiator write FIFO.  In addition to containing the FIFOs, the module also contains two state machines to control the PCI protocol specific initiator activity of the interface; an initiator read state machine and an initiator write state machine.  The DMA Interface Unit activates the Initiator Engine.

The ADC Interface Control supplies the interface logic for selecting, initiating, and reading ADC conversions.

The Serial DAC Interface Control provides the serial data stream to the serial DACs and serial Latch for setting the voltage and current DACs and associated relays.

The Channel Grouping Control block contains a 512 word (32-bit) channel mask that defines whether the current VI operation applies to a specific VI channel.

The Target Access Control block performs address decoding and control signal handshakes to allow PCI reads and writes to local registers.

- FPGA Functional Blocks

Registers

Except for the required PCI configuration registers, SEB VI FPGA registers are mapped into PCI memory space. The SEB VI FPGA occupies two independent memory areas. The configuration register PCI BASEADR0 specifies the beginning address of one 4Kbyte memory space and the configuration register PCI BASEADR1 specifies the beginning address of another 16Mbyte memory space.  The first memory block is used for internal register space, and the other 16M-memory space is used for the Digitizer Memory and history FLASH EEPROM space.  Reads and writes to the 16M byte space defined by PCI BASEADR1 are translated into reads and writes of the Digitizer memory and the FLASH EEPROM memory.  Accesses to the FLASH EEPROM memory must be 32-bit accesses.  Accessing FLASH MEMORY with cycles other than 32-bit accesses will cause incorrect operation.

    - PCI Config Registers

The PCI configuration registers map into the PCI configuration space and adhere to the format of the PCI configuration header.   Here is the PCI configuration header presented by the SEB VI FPGA during configuration cycles:

| adr | 31 |  |  |  |  |  |  | 24 | 23 |  |  |  |  |  |  | 16 | 15 |  |  |  |  |  |  | 08 | 07 |  |  |  |  |  |  | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Device ID = 0002h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Vendor ID = 10eeh |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Status |  |  |  |  |  |  |  |  |  |  | reserved |  |  |  |  | 00 |  |  |  |  |  | Command |  |  |  |  |  |  |  |  |  |
| 8 | Baseclass=11h |  |  |  |  |  |  |  | Subclass=80h |  |  |  |  |  |  |  | Program Interface=00h |  |  |  |  |  |  |  | Device Revision ID=01h |  |  |  |  |  |  |  |
| C | (BIST) |  |  |  |  |  |  |  | Header Type= |  |  |  |  |  |  |  | Latency Timer= |  |  |  |  |  |  |  | (Cache line size) |  |  |  |  |  |  |  |
| 10 | Base Address 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Base Address 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Base Address 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1C | Base Address 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Base Address 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | Base Address 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | CardBus CIS Pointer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2C | Sub-System ID = 0001h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sub-System Vendor ID = 10eeh |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | Expansion ROM Base Address |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 34 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Capabilities List Pointer=0 |  |  |  |  |  |  |  |
| 38 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3C | Maximum Latency=0 |  |  |  |  |  |  |  | Minimum Grant =0 |  |  |  |  |  |  |  | Interrupt Pin Select =01h |  |  |  |  |  |  |  | Interrupt Line Select=00h |  |  |  |  |  |  |  |

- PCI Config Register Map

- Some PCI Config Registers of Interest:

      - PCI Command Register

Byte Address: 	04h-05h

| Bit(s) | Read/ Write | Reset Value | Description/Function |
| --- | --- | --- | --- |
| 15:10 | Read | 0 | Reserved: Always read a 0. |
| 9 | Read | 0 | MFBTBEN: Master Fast Back-To-Back Enable. When active (=1) indicates a master can perform Fast Back-To-Back transactions to different PCI targets. The SEB VI FPGA does not support this feature and MFBTEN always reads as 0. |
| 8 | Read/ Write | 0 | SERRESPEN: System Error Response Enable. When both SERRESPEN and PERRESPEN are active (=1), the output pin SERR_N can be asserted. The SEB VI FPGA only asserts SERRESPEN for detected address parity errors. |
| 7 | Read | 1 | ADDRSTEP: Address Stepping. Address/Data Stepping is supported. |
| 6 | Read/ Write | 0 | PERRESPEN: Parity Error Response Enable. When active (=1) it enables the output pin PERR_N to be asserted when a PCI 36-bit even parity error is detected during the data phases of transactions. The SEB VI FPGA will assert the output pin PERR_N when PERRESPEN is active and a data parity error is detected as a target for write accesses, or as a master for read commands. |
| 5 | Read | 0 | VSNOOPEN: VGA Snoop Enable. Always read as 0. Not supported. |
| 4 | Read | 0 | MWRINEN: Memory Write and Invalidate Enable. Always read as 0. Not supported. |
| 3 | Read | 0 | SPCYCEN: Special Cycle Monitor Enable. Always read as 0. Not supported. |
| 2 | Read/ Write | 0 | MASTEREN: Master Enable. When active (=1) it enables the SEB VI FPGA to perform bus master transactions on the PCI bus. |
| 1 | Read/ Write | 0 | MSPACEEN: Memory Space Enable. When active (=1) it enables the SEB VI FPGA to respond to PCI memory space transactions. When inactive, the SEB VI FPGA will not respond to PCI memory transactions. |
| 0 | Read/ Write | 0 | ISPACEEN: IO Space Enable. When active (=1) it enables the SEB VI FPGA to respond to PCI IO space transactions. When inactive, the SEB VI FPGA will not respond to PCI IO transactions. Note: The SEB VI FPGA does not have any IO mapped registers. |

      - PCI Status Register

      - Byte Address: 	06h-07h

      - PCI BASEADR0 (Base Address 0) Register

Byte Address:	 10h

| Bit(s) | Read/ Write | Reset value | Description/Function |
| --- | --- | --- | --- |
| 31:25 | Read/ Write | 0 | BASEADR0 [31:25]: are read/write capable to indicate the mapping capability of 32M bytes of system memory space. |
| 24:4 | Read | 0 | BASEADR0 [24:4]: indicate the address space requirement. Always read as 0. |
| 3 | Read | 0 | Prefetchable: Always read as 0. The SEB VI FPGA does not support this feature. |
| 2:1 | Read | 0 | MemorySpaceAccessType[1:0]. Always read as 00. The SEB VI FPGA as a target may be located anywhere in the 32-bit address space. |
| 0 | Read | 0 | MemorySpaceIndicator: Indicates that the BASEADR0 register is used for mapping into system memory address space. Always read as 0. |

      - PCI BASEADR1 (Base Address 1) Register

      - Byte Address:	 14h

      - PCI INTPINSEL (Interrupt Pin Select) Register

      - Byte Address:	 3Dh

    - Memory Mapped Registers

The registers below have the bit definitions to control the four channels of force VI, the measurement unit. and the bus mastering DMA.  All registers are 32-bit wide.

BAR0 Registers:

| adr | 31 |  |  |  |  |  |  | 24 | 23 |  |  |  |  |  |  | 16 | 15 |  |  |  |  |  |  | 08 | 07 |  |  |  |  |  |  | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | LR | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SLOT ID[4:0] |  |  |  |  |
| 4 | Reserved Reserved Interrupt Enable[5:0] |  |  |  |  |  |  |  |  |  |  |  | Interrupt Status[3:0] |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  | Interrupt Enable[3:0] |  |  |  |
| C8 | ENTimer Counter [31:0] | TE | Timer Counter[29:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C | NOT USED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | DMA Transfer Length [31:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Host DMA Buffer Address [31:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Device Memory Address [31:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1C0 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DMA Control [5:0] |  |  |  |  |  |
| 2020 | ReservedReserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Chan Sel [3:0] |  |  |  |
| 24 | Reserved |  |  |  |  |  |  |  | Negative RAIL Voltage [7:0] |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  | Positive RAIL Voltage [7:0] |  |  |  |  |  |  |  |
| 2824 | EE | V Range |  |  | E | Compensation Compensation Reserved E |  |  |  |  | E | Reserved |  |  |  | Voltage DAC [16:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2C2C | E | I Range [3:0] |  |  |  | E | IC | Reserved |  |  | E | Reserved |  |  |  |  | Current DAC [15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3030 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DUT Connect Relay Setting |  |  |  |  |  |  |  |  |  |  |
| 3434 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E | PD | E | Sel [1:0] |  | E | VMU Rng [2:0] |  |  |
| 3838 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  | VDAC Value A [16:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3C | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  | VDAC Value B [16:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 403C | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IDAC Value A [15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | IDAC Value B [15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 | E | Reserved |  |  |  |  |  |  |  |  |  |  | Force Trigger Delay [19:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4C | E | Reserved |  |  |  |  |  |  |  |  |  |  | Pulse Width [19:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 50 | Reserved |  |  |  | Alarms |  |  |  | Reserved |  |  |  |  |  | ADC [17:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 54 | Reserved |  |  |  |  |  |  |  |  |  |  |  | Measure Trigger Delay [19:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 58 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sampling Period [15:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5C | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Samples [11:0] |  |  |  |  |  |  |  |  |  |  |  |
| 60 | Reserved |  |  |  |  |  |  |  |  |  |  |  | E | TOS Sel |  |  | E | Reserved |  |  |  |  | MTS Sel |  | E | Reserved |  |  |  | FTS Sel |  | TR |
| 64 | ADC Accumulator [31:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68 | NOT USED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6C | NOT USED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 70 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Power Status |  |  |  |
| 74 | NOT USED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 78 | NOT USED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7C | NOT USED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 80 | VDAC GAIN [31:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 84 | VDAC OFFSET [TBD:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 88 | IDAC GAIN [31:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8C | IDAC OFFSET [TBD:0] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 90 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | AWG | DIG | EEP |
| 94 | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E | TCK | TMS | TDI | TDO |

- BAR0 Registers Map

      - SLOTID  Register

Memory Map BAR0 Address:	00h

| Bit(s) | Read/ Write | Reset value | Description/Function |
| --- | --- | --- | --- |
| 31 | Write | 0 | Local Reset: Setting bit 31 will reset all local registers. This bit would be in addition to the PCI bus reset or the reset push-button reset at the board panel. |
| 30:4 | Read/ Write | 0 | Reserved |
| 4:0 | Read/ Write | 0 | SLOTID: Read back Value indicates the slot that this QVI card is plugged into. Writing to these 5 bits with the same value as read back, then the pin SELECTED (LED) would be asserted. Any “mismatched” value would de-assert the pin SELECTED. This feature is for visual verification that indeed you are accessing the write right device on the PCI bus(ses). |

FPGA / CPLD design Info:

Bits       FPGA Pins

--------   -------------

Bits 4:0   SLOTID[4:0]

      - Interrupt Enable / Status Register

      - Memory Map BAR0 Address:	04h

      - Channel Select:	No

      - Interrupt Status Register

      - Memory Map BAR0 Address:	08h

      - Channel Select:	No

      - Timer Counter Control / Status Register

      - Memory Map BAR0 Address:	08Ch

      - Channel Select:	No

      - Timer Control/Status Register

      - Memory Map BAR0 Address:	10h

      - Channel Select:	No

      - DMA Transfer Length Register

      - Memory Map BAR0 Address:	10h

      - Channel Select:	No

      - Host DMA Buffer Address Register

      - Memory Map BAR0 Address:	14h

      - Channel Select:	No

      - Device Memory Address Register

      - Memory Map BAR0 Address:	18h

      - Channel Select:	No

      - DMA Control Register

      - Memory Map BAR0 Address:	1C4h

      - Channel Select:	No

DMA Transfer Length Register

Memory Map BAR0 Address:	18h

Channel Select:	No

      - Channel_Select Register

      - Memory Map BAR0 Address:	20h

This register is used to select (enable) which channel(s) will be enabled in the subsequent access.  Since the channel selection code is not encoded, combinations can  range from a single channel to all channels.  This register would remain as programmed until the next update.  So this register has to be programmed first if access to “channel-wise register” is desired.  This register can be considered as a page select register.

Note:  The entry of  “Channel Select: YES/NO” for registers below, denotes:

An entry of YES means there are actually as many registers with that address as there are channels.  In this design there are 4.

e.g. The Voltage_Force Register Voltage Force Register shows Channel Select to be YES, so there should be 4 such registers, each enabled by not only the Address decode but by their respective Channel Select bit.

      - PowerAmp Rail Voltage Register

      - Memory Map BAR0 Address:	24h

      - Channel Select:	YES

      - Total Number of Registers : 2  (Channel Pairs)

This register controls the supply voltages for the Power Amp of each channel pair.  The reason for this control is to reduce Power, the Power Amp dissipates.  The Power is calculated to be (VP – Vout) * Current.  So the maximum power would be dissipated at lower voltages.

Channel Select bits 0 or 1 enables loading of the 8bit DAC for Channels 1 and 2’s Rail Supplies.

Channel Select bits 2 or 3 enables loading of the 8bit DAC for Channels 3 and 4’s Rail Supplies.

FPGA Pins

DC12_8bit_SDIN    // Channel 1&2 Rail Voltage Serial DAC’s DataIn

DC12_8bit_SCLK    // Channel 1&2 Rail Voltage Serial DAC’s Serial Clock

DC12_8bit_SYNCA   // Channel 1&2 Rail Voltage Serial DAC’s Load Enable

DB34_8bit_SDIN    // Channel 3&4 Rail Voltage Serial DAC’s DataIn

DB34_8bit_SCLK    // Channel 3&4 Rail Voltage Serial DAC’s Serial Clock

DB34_8bit_SYNCB   // Channel 3&4 Rail Voltage Serial DAC’s Load Enable

      - Voltage_Force Register

      - Memory Map BAR0 Address:	284h

      - Channel Select:	YES

      - Total Number of Registers : 4

This register forces the channel output to a desired voltage.  This register is a multi-function register that contains not only the DAC value but also the compensation select to compensate for various loads.  There are enable bits to update the range selection and the compensation selections.  When these enable bits are NOT set, then Software the Host doesn’t have to update the range and compensation selection every time the DAC value is updated.

Bit 20 enables the user to use the DAC Instruction Memory in lieu of writing to this register to load the VDAC.  Again Bit21 is the enable to update Bit20.

FPGA / CPLD design Info:

FPGA issue a serial transfer to the CPLD when accessing this register.

Bits

30:28    a b   VPP   VPM    KRVP  KRVM

=======  ========================================================

0x0      1 0   x     x      0    0   	// -1V to +1V

0x1      0 0   x     x      0    0   	// -5V to +5V

0x2      1 1   +28   -28    1    1   	// -10V to +10V  //DEFAULT

0x3      0 1   +577   x      1    0    // -7V to +50V

0x4      0 1   x     -577    0    1    // -50V to +7V

0x5      0 1   +40   -28    1    1    // -22V to +35V

0x6      0 1   +28   -40    1    1    // -35V to +22V

0x7      0 1   +30   -30    1    1    // -25V to +25V

CPLD (CpxU1) PINS:

CHx_VSW1 = a

CHx_VSW0 = b

VDAC_P_CHx_N = !Bit16

FPGA (U17) PINS:

MVPx_ONN = !KRVM

PVPx_ONN = !KRVP

Serial DACs:

VPP = DACA of 8-bit Serial DAC

VPM = DACB of 8-bit Serial DAC

Bits

26:25   d e f

=====   =====

0       0 0 1   // 24.545K Ohm ... Fastest

1*      1 0 1   // 133K Ohm ............DEFAULT??

2       0 1 0   // 538.784K Ohm

3       0 0 0   // 4.99M Ohm ... slowest

CPLD (CpxU1) PINS:

CHx_50_RANGE_N = d

CHx_5_10_RANGE_N = e

CHx_SLEW_N = f

Bits

24:22	 g h k

=====  =====

0      0 0 0   // 0.0047 uF ... Fastest

1      0 0 1   // 0.0094 uF

2      0 1 0   // 0.1047 uF

3*     0 1 1   // 0.1094 uF .....DEFAULT??

4      1 0 0   // 0.2247 uF

5      1 0 1   // 0.2294 uF

6      1 1 0   // 0.3247 uF

7      1 1 1   // 0.3294 uF ... slowest   DEFAULT??

CPLD (CpxU1) PINS:

CHx_COMP[3]_N = !g

CHx_COMP[2]_N = !h

CHx_COMP[1]_N = !k

Voltage DACs: Serial DAC AD5544

Bits 15:0 => D[15:0} (see below)

A1 A0

=====  ================

0  0   CHA Voltage DAC

0  1   CHA Current DAC

1  0   CHB Voltage DAC

1  1   CHB Current DAC  where A=CH1, CH3, B=CH2, CH4

CPLD (CpxU1) PINS:

DAC_CLK

DAC_CS_N

DAC_RS_N

DAC_SDI ……..{A1,A0,D[15:0]}

DAC_LDAC_N

PowerAmp Rail Voltage Register

Memory Map BAR0 Address:	28h

Channel Select:	YES

This register controls the supply voltages for the Power Amp of each channel pair.  The reason for this control is to reduce Power, the Power Amp dissipates.  The Power is calculated to be (VP – Vout) * Current.  So the maximum power would be dissipated at lower voltages.

Channel Select bits 0 or 1 enables loading of the 8bit DAC for Channels 1 and 2’s Rail Supplies.

Channel Select bits 2 or 3 enables loading of the 8bit DAC for Channels 3 and 4’s Rail Supplies.

FPGA Pins

DC12_8bit_SDIN    // Channel 1&2 Rail Voltage Serial DAC’s DataIn

DC12_8bit_SCLK    // Channel 1&2 Rail Voltage Serial DAC’s Serial Clock

DC12_8bit_SYNCA   // Channel 1&2 Rail Voltage Serial DAC’s Load Enable

DB34_8bit_SDIN    // Channel 3&4 Rail Voltage Serial DAC’s DataIn

DB34_8bit_SCLK    // Channel 3&4 Rail Voltage Serial DAC’s Serial Clock

DB34_8bit_SYNCB   // Channel 3&4 Rail Voltage Serial DAC’s Load Enable

      - Current_Force Register

      - Memory Map BAR0 Address:	2Ch

      - Channel Select:	YES

      - Total Number of Registers : 4

This register forces the channel output to a specified current.  This register is a multi-function register that contains not only the DAC value but also the compensation select to compensate for various loads.  There are enable bits to update the range selection and the compensation selections.  When these enable bits are NOT set, then Software doesn’t have to update the range and compensation selection every time the DAC value is updated.

Bit 24 enables the user to use the DAC Instruction Memory in lieu of writing to this register to load the IDAC.  Again Bit 25 is the enable to update Bit24.

FPGA/CPLD design Info:

FPGA issue a serial transfer to the CPLD when accessing this register.

Bits

30:27   a b c d e f g h j   k l m n

======  ===========================

0xA     1 0 0 0 0 0 0 0 0   0 0 0 1  5A

0x9     0 1 0 0 0 0 0 0 0   0 0 0 1  1A

0x8     0 0 1 0 0 0 0 0 0   0 0 0 1  100MA

0x7     0 0 0 1 0 0 0 0 0   0 1 1 1  10MA

0x6     0 0 0 0 1 0 0 0 0   0 1 1 1  1MA *  Default

0x5     0 0 0 0 0 1 0 0 0   0 1 1 1  100UA

0x4     0 0 0 0 0 0 1 0 0   1 1 0 0  10UA

0x3     0 0 0 0 0 0 0 1 0   1 1 0 0  1UA

0x2     0 0 0 0 0 0 0 0 1   1 1 0 0  100NA

0x1     0 0 0 0 0 0 0 0 0   1 1 0 0  10NA

CPLD (CpxU1)PINS:

CHx_5A = a

CHx_1A = b

CHx_100MA = c

CHx_10MA = d

CHx_1MA = e

CHx_100UA = f

CHx_10UA = g

CHx_1UA = h

CHx_100NA = j

CHx_5A-100UA = k

CHx_10UA-10NA = l

CHx_10MA-100UA_NSEN = m

CHx_10MA-10NA_PSEN = n

Bit25   I_COMP_

======  ========

0       1          Fast   // C=0.047 uF

1*      0          Slow   // C=0.197 uF  DEFAULT

CPLD (CpxU1) PIN:

CHx_ICOMP_N

Current DACs: Serial DAC AD5544

Bits 15:0 => D[15:0} (see below)

A1 A0

=====  ================

0  0   CHA Voltage DAC

0  1   CHA Current DAC

1  0   CHB Voltage DAC

1  1   CHB Current DAC  where A=CH1, CH3, B=CH2, CH4

CPLD (CpxU1) PINS:

DAC_CLK

DAC_CS_N

DAC_RS_N

DAC_SDI ……..{A1,A0,D[15:0]}

DAC_LDAC_N

      - DUT_Connect Register

      - Memory Map BAR0 Address:	30h

      - Channel Select:	YES

      - Total Number of Registers : 4

This register controls how the channel is connected to the DUT and to other channels for special modes. A special mode is when performing the GFS tests where one channel controls another’s current loop.  In this case, bits 3 and 4 are to be programmed.  In the case of needing to double up the current, simply connect the 2 channels (Odd and Even pair) to the DUT AND program bit 6 of the Even channel to a 1.  This will cause the Even channel to follow the Odd channel, voltage wise.

FPGA/CPLD Design Info:

FPGA issue a serial transfer to the CPLD when accessing this register.

Bit0       ON

=======    ===        ===============================

0*         0          Output Relay open ... DEFAULT

1          1          Output Relay Closed

CPLD U11 PIN:

CHn_ON = ON

Bit1     INT    Operation

=====    ===    =================

0        0      Disable Remote Sense, use internal local sense

1*       1      Enable Remote Sense .... DEFAULT

CPLD U11 PIN:

CHn_INT = INT

Bit 4:3   LOCAL    EXT

========  ======   =====   ==========================================

00*       1       0       Normal Usage, internal Loop ... DEFAULT

01        1       0       Normal Usage, internal Loop

10        0       1       Channel is a slave, disable internal loop

11        1       1       Channel is a master, output ctrl to ext pin

CPLD(CpxU1)PINS:

CHx_LOCAL_IMON_N = !LOCAL  @#$% Check Polarity

CHx_SEL_X_IMON_N = !EXT    @#$% Check Polarity

Bit6   Follow

====   ======

0*        0     Normal Operation.

1         1     Even Channel(s) to FOLLOW Odd Channel(s).

CPLD(CpxU1)PIN:

VFOLLOW_2 = !Follow

CPLD U11 PINS:

CHx_KELVIN_HI = Bit9

CHx_KELVIN_LO = Bit8

      - VMU_Input Register

      - Memory Map BAR0 Address:	34h

      - Channel Select:	YES

      - Total Number of Registers : 4

This register selects both the range and inputs to the ADC.  Both selects have their own enable bit, making them totally independent when writing to this register.

FPGA/CPLD Design Info:

|-VMU-|   |-ADC-|

1

0 5 2 1   1

Bits    0 0 0 0   0 5 2 1

2:0     V V V V   V V V V   RANGE

====    =================   =====

0x6     1 0 0 0   1 0 0 0   100V

0x5*    0 1 0 0   1 0 0 0   50V ….. Default

0x4     0 0 1 0   1 0 0 0   20V

0x3     0 0 0 1   1 0 0 0   10V

0x2     0 0 0 1   0 1 0 0   5V

0x1     0 0 0 1   0 0 1 0   2V

0x0     0 0 0 1   0 0 0 1   1V

CPLD U11 PINS:

CH[n]_VMU_100V_SEL = VMU_100V

CH[n]_VMU_50V_SEL = VMU_50V

CH[n]_VMU_20V_SEL = VMU_20V

CH[n]_VMU_10V_SEL = VMU_10V

CH[n]_ADC_RNG_10V = !ADC_10V

CH[n]_ADC_RNG_5V = !ADC_5V

CH[n]_ADC_RNG_2V = !ADC_2V

CH[n]_ADC_RNG_1V = !ADC_1V

D K K C

SEL         U H L A

COD     V I T I O L     SELECTS

=====  ==============   ===============================

0x0     1 0 0 0 0 0     VMeasure ... Internal

0x1     0 1 0 0 0 0     IMeasure ... Internal

0x2     0 0 1 0 0 0     DUT DVM  ... External Pin Measure (Load Board)

0x3     0 0 0 1 0 0     Kelvin High Sense ... Hi Sense connect test

0x4     0 0 0 0 1 0     Kelvin Low Sense .... Lo Sense connect test

0x5     0 0 0 0 0 1     CAL resistors

CPLD U11 PINS:

CH[n]_VMU_VI_E_SEL = V

CH[n]_VMU_VI_I_SEL = I

CH[n]_VMU_DUT_SEL = DUT

CH[n]_VMU_KHI_SEL = KHI

CH[n]_VMU_KLO_SEL = KLO

CH[n]_VMU_CAL_SEL = CAL

CPLD U11 Pin:

CHn_ADC_PD = bit8;

      - VDAC_A Register

Memory Map BAR0 Address:	38h

Channel Select:	YES

Total Number of Registers : 4

| Bit(s) | Read/ Write | Reset value | Description/Function |
| --- | --- | --- | --- |
| 31:17 | - | - | Reserved |
| 16:0 | Read/Write | 0? | Value A: This 16 bit plus sign number will be loaded into the respective channel’s VDAC when the trigger condition is met and the trigger delay timer has expired. |

This register holds the initial Voltage that Channel will be set to when the trigger condition is met and the trigger delay timer has expired.  See the following diagram.  This register holds the V1 value.

FPGA Design Info:

The timer works off 10MHz (100MHz divide by 10) Clock.

      - VDAC_B Register

      - Memory Map BAR0 Address:	3Ch

      - Channel Select:	YES

      - Total Number of Registers : 4

This register holds the second Voltage that Channel will be set to when the pulse width condition is met. See the above diagram.  This register holds the V2 value.

Note: V2 can be equaled to V1 if a DC or non-switching is desired.

      - IDAC_A Register

      - Memory Map BAR0 Address:	403Ch

      - Channel Select:	YES

      - Total Number of Registers : 4

This register holds the initial Voltage Value for IDAC that Channel will be set to when the trigger condition is met and the trigger delay timer has expired.  See the following diagram.  This register holds the V1 value.

FPGA Design Info:

The timer works off 10MHz (100MHz divide by 10) Clock.

      - IDAC_B Register

      - Memory Map BAR0 Address:	44h

      - Channel Select:	YES

This register holds the second Voltage Value for IDAC that Channel will be set to when the pulse width condition is met. See the above diagram.  This register holds the V2 value.

Note: V2 can be equaled to V1 if a DC or non-switching is desired.

      - Force Trigger Delay Register

      - Memory Map BAR0 Address:	480h

      - Channel Select:	NOYES

This register holds the count (based on 100ns clocks) for the Trigger Delay timer.  This timer will start counting when the Trigger condition is met AND bit 31=1. A value of zero (0) effectively set the VDAC and IDAC to the programmed values stored at VDAC_A and IDAC_A registers IMMEDIATELY after the trigger occurs.

After the delay time expired, this timer will signal to the WIDTH timer to start the count down.

FPGA Design Info:

The timer works off 10MHz (100MHz divide by 10) Clock.

      - Pulse Width Register

      - Memory Map BAR0 Address:	4C4h

      - Channel Select:	YESNO

This register holds the count (based on 100ns clocks) for the “Pulse Width”.   This timer will start as soon as the Trigger Delay timer has reached its programmed count.  After this timer reached its programmed count, it will trigger the loading of the values stored at the VDAC_B and IDAC_B registers to the respective DACs.  A value of zero (0) effectively set the DACs immediately after the Trigger Delay timer reached its programmed time.

So if Trigger Delay Register = 0, this Pulse Width Register = 0 and VA=VB, this means that the DACs will be loaded as soon as the trigger condition is met.

      - ADC_Measure Register

      - Memory Map BAR0 Address:	504Ch

      - Channel Select:	YES

      - Total Number of Registers : 4

This register is a Read-Only register.  This register returns the ADC value including sign, Power Module Status, Attenuator Over-Range, ADC Over-Range  and Current Clamped Mode Status. If any of the bits 24-26, the reading of the ADC reading should be ignore as something is wrong with either the Hardware or wrong range were selected.

FPGA/CPLD Design Info

The ADC used is an Analog Device AD7674.  Reading this register will sample the ADC and return this DWORD to the PC.

Bit 24: This is the OR of PS_IOG_[1:4] at the U11 inputs. The OR’ed function has to be sent back to the FPGA to merge with this register.  So reading this register should initiate a read from the CPLD U11.

Bit 25:

IN_OVRNGx

Bit 26:

OUT_OVRNGx

Bit 27:

CHx_VCLAMPED aka B12_DATA_OUT2, B12_DATA_OUT1, C34_DATA_OUT1, C34_DATA_OUT2.  (FPGA)

      - Measure Trigger Delay Register

      - Memory Map BAR0 Address:	540h

      - Channel Select:	YESNO

This register holds the count (based on 100ns clocks) for the ADC Trigger Delay timer.  This timer will start counting when the Trigger condition is met. A value of zero (0) effectively start the acquisition process IMMEDIATELY after the trigger occurs.

The ADC used is an Analog Device AD7674.  When the trigger condition is met, the FPGA sends request for sample through these pins:

ADC_CNVSTx_N (output)

ADC_CLKx (output)

FPGA monitors ADC_BUSYx (input) and when it’s ready, reads value from ADC_SDATAx (input)

      - Sampling Period Register

      - Memory Map BAR0 Address:	584h

      - Channel Select:	YESNO

This register holds the period count (based on 100ns clocks) for sampling of the ADC.   This period counter will start as soon as the Trigger Delay timer has reached its programmed count.  When the programmed number of samples is met, this period counter will stop.

      - Samples Register

      - Memory Map BAR0 Address:	5C8h

      - Channel Select:	YESNO

This register holds the number of samples desired.  As soon as the measure trigger delay is done, this counter will start counting the number of acquisitions.  This counter is enabled (to count)  by the sampling period counter.

      - Start Trigger Register

      - Memory Map BAR0 Address:	605Ch

      - Channel Select:	NO

This register controls the triggering event for both the DAC and ADC.  This register also selects the trigger out source.

FPGA Design Info

U17 Pins:

TR1 = Trigger Out, sending a trigger to the trigger bus based on bits [18:16] selection

TRI = Trigger Bus IN

      - ADC Accumulator Register

      - Memory Map BAR0 Address:	640h

      - Channel Select:	YES

      - Total Number of Registers : 4

This register reads back the total sum of the ADC measurements.  The value of the ADC is 1 signed bit plus 17 bits of data. Software will read this register and divide it by the number of samples specified in the ADC Samples register.  The sample time frequency is specified via Sampling Period Register, a fixed Frequency based on the 100MHz time base. The resolution is 100ns.   The Maximum samples would be 64k.

      - ADC Digitizing Memory

      - Memory Map BAR1 Address:	xxx 0 h through  yyy 3ff h … 1K deep.

      - Channel Select:	YES

      - Total Number of Registers : 4

      - Enable: BAR0 addr 90h

This memory is 22 bits wide and 1k deep. The address space occupied by this memory will be 4k (reserved).  There’s no address pointer for this memory, the entire memory is mapped into the PCI memory space.

The Trigger Start will reset this memory’s pointer to zero.

      - Board Info EPROM Memory

      - Memory Map BAR1 Address:	nnn 0 h through  7fffmmm h … 32k deep.

      - Channel Select:	NO

      - Enable: BAR0 addr 90h

The EEPROM is an Atmel AT25P1024, which is a 128k by 8 SPI Serial EEPROM.  The write function is limited to a 128- byte page write only.  The read function will be limited to a 4 byte access only.  There will be significant time delay after each page write.

FPGA DESIGN INFO:

The AT25P1024 supports SPI1/3 protocol. Writing to this register should cause a serial transfer of 1024 bits to the pins below.  Reading one word address would cause a serial transfer of 32 bits from the EEPROM.

FPGA Pins:

PBEP_SI, PBEP_SCK, PBEP_CS_N, PBEP_SO

      - Power Status Register

      - Memory Map BAR0 Address:	70h

      - Channel Select:	No

This register reads back the status of the power modules that power the Output Amplifiers for each channel pair. This is a read-only register.

FPGA/CPLD Design Info:

U11 Pins:

PS_IOG[1] = bit0  // +VPB OK … (+VPB = +28V to +57V Rail Supply for CH1 & CH2);

PS_IOG[2] = bit1  // -VPB OK … (-VPB = -28V to -57V Rail Supply for CH1 & CH2);

PS_IOG[3] = bit2  // +VPC OK … (+VPC = +28V to +57V Rail Supply for CH3 & CH4);

PS_IOG[4] = bit3  // -VPC OK … (-VPC = -28V to -57V Rail Supply for CH3 & CH4);

These CAL registers may be postponed until a later date.

      - Channel n Voltage DAC Gain Register

      - Memory Map BAR0 Address:	80h

      - Channel Select:	YES

      - Total Number of Registers : 4 * 8 Ranges

      - Channel n Voltage DAC Offset Register

      - Memory Map BAR0 Address:	84h

      - Channel Select:	YES

      - Total Number of Registers : 4 * 8 Ranges

      - Channel n Current DAC Gain Register

      - Memory Map BAR0 Address:	88h

      - Channel Select:	YES

      - Total Number of Registers : 4 * 10 Ranges

      - Channel n Current DAC Offset Register

      - Memory Map BAR0 Address:	8Ch

      - Channel Select:	YES

      - Total Number of Registers : 4 * 10 Ranges

Channel n Voltage Measure Gain Register

Memory Map BAR0 Address:	90h

Channel Select:	YES

      - Channel n Voltage Measure Offset Register

      - Memory Map BAR0 Address:	94h

      - Channel Select:	YES

      - Channel n Current Measure Gain Register

      - Memory Map BAR0 Address:	98h

      - Channel Select:	YES

      - Channel n Current Measure Offset Register

      - Memory Map BAR0 Address:	9Ch

      - Channel Select:	YES

      - Channel n DUT Voltage Measure Gain Register

      - Memory Map BAR0 Address:	A0h

      - Channel Select:	YES

      - Channel n DUT Voltage Measure Offset Register

      - Memory Map BAR0 Address:	A4h

      - Channel Select:	YES

      - BAR1 Memory Type Register

      - Memory Map BAR0 Address:	90h

      - Channel Select:	No

      - This register selects the memory that would be accessed through the BAR1 address space.  Since all memory are expandable it would be difficult to avoid any address offset adjust when reserving the address space.  So a memory type select register would give an additional decoding of the memories

| Bit(s) | Read/ Write | Reset value | Description/Function |
| --- | --- | --- | --- |
| 31:3 | - |  | Reserved |
| 2 | Read/ Write | 0 | Reserved for AWG |
| 1 | Read/ Write | 0 | Digitizing Memory: 1 = BAR1 is used for Digitizer memory access |
| 0 | Read/Write | 0 | Info EEPROM Memory: 1 = BAR1 is used for Info EEPROM access |

====================================================================

      - Image Update Register

      - Memory Map BAR0 Address:	9B40h

      - Channel Select:	No

      - This register drives the a JTAG port on the serial Config eeproms to allow in-system upgrade of the FPGA image.  The TCK, TMS, and TDI bit drive FPGA pins that connect to the serial eeprom.  TDO connects to an input pin on the FPGA.

| Bit(s) | Read/ Write | Reset value | Description/Function |
| --- | --- | --- | --- |
| 317:53 | Read | 0 | Reserved |
| 4 | Read | 0 | Image_Update_Enable: Set (=1) allows the updating of the FPGA PROM image. Otherwise the TCK,TMS, and TDI are tri-state (normal operation). |
| 3 | Read/ Write | 0 | TCK: The value of this bit is driven onto the TCK_EEPROM output pin for programming FPGA serial eeprom images. |
| 2 | Read/ Write | 0 | TMS: The value of this bit is driven onto the TMS_EEPROM output pin for programming FPGA serial eeprom images. |
| 1 | Read/ Write | 0 | TDI: The value of this bit is driven onto the TDI_EEPROM output pin for programming FPGA serial eeprom images. |
| 0 | Read | 0 | TD0: reflects the value on the TDO_EEPROM input pin. |

- HARDWARE DESIGN INFO

  - FPGA – U17

    - FPGA Pin Map

    - FPGA Pin Description

    - FPGA (U17) UCF

    - #

    - # CLOCKS

    - #

    - NET "100MHZ_CK1" LOC = "A10";

    - NET "100MHZ_CK1_N" LOC = "B10";

    - NET "RCLK1" LOC = "F12";

    - NET "RCLK1_N" LOC = "F11";

    - #

    - # PCI BUS

    - #

    - NET "PCI_CLK2" LOC = "H1";

    - NET "AD[0]" LOC = "E1";

    - NET "AD[1]" LOC = "D1";

    - NET "AD[2]" LOC = "D2";

    - NET "AD[3]" LOC = "E2";

    - NET "AD[4]" LOC = "E3";

    - NET "AD[5]" LOC = "G5";

    - NET "AD[6]" LOC = "G6";

    - NET "AD[7]" LOC = "E4";

    - NET "AD[8]" LOC = "D3";

    - NET "AD[9]" LOC = "H7";

    - NET "AD[10]" LOC = "H8";

    - NET "AD[11]" LOC = "C1";

    - NET "AD[12]" LOC = "C2";

    - NET "AD[13]" LOC = "G7";

    - NET "AD[14]" LOC = "F7";

    - NET "AD[15]" LOC = "E5";

    - NET "AD[16]" LOC = "C5";

    - NET "AD[17]" LOC = "C7";

    - NET "AD[18]" LOC = "B7";

    - NET "AD[19]" LOC = "G8";

    - NET "AD[20]" LOC = "F8";

    - NET "AD[21]" LOC = "G9";

    - NET "AD[22]" LOC = "G10";

    - NET "AD[23]" LOC = "C6";

    - NET "AD[24]" LOC = "B6";

    - NET "AD[25]" LOC = "F9";

    - NET "AD[26]" LOC = "E9";

    - NET "AD[27]" LOC = "A5";

    - NET "AD[28]" LOC = "A6";

    - NET "AD[29]" LOC = "E10";

    - NET "AD[30]" LOC = "F10";

    - NET "AD[31]" LOC = "D7";

    - NET "CBE_L[0]" LOC = "E6";

    - NET "CBE_L[1]" LOC = "D6";

    - NET "CBE_L[2]" LOC = "A9";

    - NET "CBE_L[3]" LOC = "D5";

    - NET "DEVSEL_L" LOC = "B4";

    - NET "FRAME_L" LOC = "A3";

    - NET "GNT_N1" LOC = "A7";

    - NET "IDSEL1" LOC = "D8";

    - NET "ITB_N" LOC = "C10";

    - NET "IRDY_L" LOC = "B3";

    - NET "PAR" LOC = "B9";

    - NET "PERR_L" LOC = "D4";

    - NET "REQ_N1" LOC = "C8";

    - NET "RST_L" LOC = "D10";

    - NET "SERR_L" LOC = "E7";

    - NET "STOP_L" LOC = "C4";

    - NET "TRDY_L" LOC = "A4";

    - #

    - # SLOT ID & LED TO INDICATE SLOT IS ACCESSED

    - #

    - NET "SELECTED1" LOC = "G4";

    - NET "SLOTID0" LOC = "H6";

    - NET "SLOTID1" LOC = "H5";

    - NET "SLOTID2" LOC = "G2";

    - NET "SLOTID3" LOC = "G1";

    - NET "SLOTID4" LOC = "H4";

    - #

    - # EEPROM ITERFACE

    - #

    - NET "PBEP_CS_N" LOC = "U6";

    - NET "PBEP_SCK" LOC = "U7";

    - NET "PBEP_SI" LOC = "T8";

    - NET "PBEP_SO" LOC = "U5";

    - #

    - # VI ITERFACE

    - #

    - NET "ADC_BUSY1" LOC = "Y5";

    - NET "ADC_BUSY2" LOC = "AE3";

    - NET "ADC_BUSY3" LOC = "Y9";

    - NET "ADC_BUSY4" LOC = "AA10";

    - NET "ADC_CLK1" LOC = "AC4";

    - NET "ADC_CLK2" LOC = "AF4";

    - NET "ADC_CLK3" LOC = "AE7";

    - NET "ADC_CLK4" LOC = "AC9";

    - NET "ADC_CNVST1" LOC = "AB4";

    - NET "ADC_CNVST2" LOC = "AE4";

    - NET "ADC_CNVST3" LOC = "AD7";

    - NET "ADC_CNVST4" LOC = "AB9";

    - NET "ADC_SDATA1" LOC = "Y6";

    - NET "ADC_SDATA2" LOC = "AF3";

    - NET "ADC_SDATA3" LOC = "AA9";

    - NET "ADC_SDATA4" LOC = "Y10";

    - NET "IN_OVRNG1" LOC = "AC5";

    - NET "IN_OVRNG2" LOC = "AF6";

    - NET "IN_OVRNG3" LOC = "AF8";

    - NET "IN_OVRNG4" LOC = "AF9";

    - NET "OUT_OVRNG1" LOC = "AB5";

    - NET "OUT_OVRNG2" LOC = "AF5";

    - NET "OUT_OVRNG3" LOC = "AF7";

    - NET "OUT_OVRNG4" LOC = "AE9";

    - NET "DB34_8BIT_SCLK" LOC = "AC3";

    - NET "DB34_8BIT_SDI" LOC = "AD3";

    - NET "DB34_8BIT_SYNCB" LOC = "AC6";

    - NET "DC12_8BIT_SCLK" LOC = "AB7";

    - NET "DC12_8BIT_SDI" LOC = "AC7";

    - NET "DC12_8BIT_SYNCA" LOC = "AC2";

    - NET "MVP4_ONN" LOC = "AD8";

    - NET "PVP4_ONN" LOC = "AC8";

    - NET "MVP3_ONN" LOC = "AA8";

    - NET "PVP3_ONN" LOC = "Y8";

    - NET "MVP2_ONN" LOC = "AD5";

    - NET "PVP2_ONN" LOC = "AD4";

    - NET "MVP1_ONN" LOC = "AD2";

    - NET "PVP1_ONN" LOC = "AD1";

    - NET "CH4_VCLAMPED" LOC = "AA3";

    - NET "CH3_VCLAMPED" LOC = "AA4";

    - NET "CH2_VCLAMPED" LOC = "Y3";

    - NET "CH1_VCLAMPED" LOC = "Y4";

    - #  SPARE # NET "C34_DATA_OUT3" LOC = "W5";

    - #  SPARE # NET "B12_DATA_OUT3" LOC = "Y1";

    - #

    - # CP1U1 ITERFACE

    - #

    - NET "B12_SCLK" LOC = "V5";

    - NET "B12_SDATA_I" LOC = "V6";

    - NET "B12_STRB1" LOC = "W4";

    - NET "B12_STRB2" LOC = "W3";

    - NET "B12_SDATA_OUT" LOC = "Y2";

    - #

    - # CP2U1 ITERFACE

    - #

    - NET "C34_SCLK" LOC = "W1";

    - NET "C34_SDATA_I" LOC = "W2";

    - NET "C34_STRB1" LOC = "W7";

    - NET "C34_STRB2" LOC = "V7";

    - NET "C34_SDATA_OUT" LOC = "W6";

    - #

    - # U11 CPLD

    - #

    - NET "CPLD_SCLK" LOC = "AB2";

    - NET "CPLD_SDATAI" LOC = "AB3";

    - NET "CPLD_SDATAOUT" LOC = "AB1";

    - NET "CPLD_STRB" LOC = "AA1";

    - #

    - # UNUSED ANALOG ITERFACE SIGNALS

    - #

    - # NET "PROTO_DIFF_N0" LOC = "AE12";

    - # NET "PROTO_DIFF_N1" LOC = "AB10";

    - # NET "PROTO_DIFF_N2" LOC = "AC17";

    - # NET "PROTO_DIFF_N3" LOC = "AE13";

    - # NET "PROTO_DIFF0" LOC = "AF12";

    - # NET "PROTO_DIFF1" LOC = "AC10";

    - # NET "PROTO_DIFF2" LOC = "AB17";

    - # NET "PROTO_DIFF3" LOC = "AE14";

    - #

    - # HEADER TEST POIT

    - #

    - # NET "TEST_HDR_BY2_0" LOC = "J7";

    - # NET "TEST_HDR_BY2_1" LOC = "J6";

    - # NET "TEST_HDR_BY2_2" LOC = "J5";

    - # NET "TEST_HDR_BY2_3" LOC = "J4";

    - # NET "TEST_HDR_BY2_4" LOC = "K7";

    - # NET "TEST_HDR_BY2_5" LOC = "K6";

    - # NET "TEST_HDR_BY2_6" LOC = "J2";

    - # NET "TEST_HDR_BY2_7" LOC = "L7";

    - # NET "TEST_HDR_BY3_0" LOC = "L6";

    - # NET "TEST_HDR_BY3_1" LOC = "K5";

    - # NET "TEST_HDR_BY3_2" LOC = "K4";

    - # NET "TEST_HDR_BY3_3" LOC = "K3";

    - # NET "TEST_HDR_BY3_4" LOC = "K2";

    - # NET "TEST_HDR_BY3_5" LOC = "L4";

    - # NET "TEST_HDR_BY3_6" LOC = "L3";

    - # NET "TEST_HDR_BY3_7" LOC = "M8";

    - # NET "SYS_RESET_I" LOC = "L1";

    - # NET "PASS" LOC = "L8";

    - #

    - # SYNCHRONIZATION AND TRIGGER ... TO Other FPGA

    - #

    - # NET "SDLL" LOC = "AD13";

    - # NET "TR0" LOC = "AC15";

    - NET "TR1" LOC = "AC16";    #TRIGGER 1 (Output)

    - NET "TRI" LOC = "AC13";    #TRIGGER IN (Input)

    - #

    - # DDR2 DIMM ITERFACE

    - #

    - # NET "DDR2_A0" LOC = "AC22";

    - # NET "DDR2_A1" LOC = "Y22";

    - # NET "DDR2_A10" LOC = "E20";

    - # NET "DDR2_A11" LOC = "D20";

    - # NET "DDR2_A12" LOC = "A18";

    - # NET "DDR2_A2" LOC = "AA26";

    - # NET "DDR2_A3" LOC = "AB26";

    - # NET "DDR2_A4" LOC = "AC26";

    - # NET "DDR2_A5" LOC = "R26";

    - # NET "DDR2_A6" LOC = "P22";

    - # NET "DDR2_A7" LOC = "L26";

    - # NET "DDR2_A8" LOC = "M24";

    - # NET "DDR2_A9" LOC = "C26";

    - # NET "DDR2_BA0" LOC = "AB22";

    - # NET "DDR2_BA1" LOC = "AB23";

    - # NET "DDR2_CAS_N" LOC = "AC23";

    - # NET "DDR2_CK" LOC = "AD25";

    - # NET "DDR2_CK_N" LOC = "AD26";

    - # NET "DDR2_CKE" LOC = "AA23";

    - # NET "DDR2_CS_N" LOC = "AD22";

    - # NET "DDR2_DM0" LOC = "B24";

    - # NET "DDR2_DM1" LOC = "G19";

    - # NET "DDR2_DM2" LOC = "H22";

    - # NET "DDR2_DM3" LOC = "J21";

    - # NET "DDR2_DM4" LOC = "M19";

    - # NET "DDR2_DM5" LOC = "N21";

    - # NET "DDR2_DM6" LOC = "T26";

    - # NET "DDR2_DM7" LOC = "V21";

    - # NET "DDR2_DM8" LOC = "AC25";

    - # NET "DDR2_DQ0" LOC = "D17";

    - # NET "DDR2_DQ1" LOC = "C20";

    - # NET "DDR2_DQ10" LOC = "E18";

    - # NET "DDR2_DQ11" LOC = "E21";

    - # NET "DDR2_DQ12" LOC = "D22";

    - # NET "DDR2_DQ13" LOC = "C22";

    - # NET "DDR2_DQ14" LOC = "A22";

    - # NET "DDR2_DQ15" LOC = "A21";

    - # NET "DDR2_DQ16" LOC = "F19";

    - # NET "DDR2_DQ17" LOC = "E23";

    - # NET "DDR2_DQ18" LOC = "E22";

    - # NET "DDR2_DQ19" LOC = "F20";

    - # NET "DDR2_DQ2" LOC = "B20";

    - # NET "DDR2_DQ20" LOC = "H20";

    - # NET "DDR2_DQ21" LOC = "G20";

    - # NET "DDR2_DQ22" LOC = "F24";

    - # NET "DDR2_DQ23" LOC = "F23";

    - # NET "DDR2_DQ24" LOC = "H21";

    - # NET "DDR2_DQ25" LOC = "E25";

    - # NET "DDR2_DQ26" LOC = "E24";

    - # NET "DDR2_DQ27" LOC = "G24";

    - # NET "DDR2_DQ28" LOC = "H24";

    - # NET "DDR2_DQ29" LOC = "H23";

    - # NET "DDR2_DQ3" LOC = "B18";

    - # NET "DDR2_DQ30" LOC = "G26";

    - # NET "DDR2_DQ31" LOC = "G25";

    - # NET "DDR2_DQ32" LOC = "J20";

    - # NET "DDR2_DQ33" LOC = "K22";

    - # NET "DDR2_DQ34" LOC = "K21";

    - # NET "DDR2_DQ35" LOC = "J26";

    - # NET "DDR2_DQ36" LOC = "L21";

    - # NET "DDR2_DQ37" LOC = "L20";

    - # NET "DDR2_DQ38" LOC = "K24";

    - # NET "DDR2_DQ39" LOC = "K23";

    - # NET "DDR2_DQ4" LOC = "C19";

    - # NET "DDR2_DQ40" LOC = "N19";

    - # NET "DDR2_DQ41" LOC = "L24";

    - # NET "DDR2_DQ42" LOC = "L23";

    - # NET "DDR2_DQ43" LOC = "M25";

    - # NET "DDR2_DQ44" LOC = "M23";

    - # NET "DDR2_DQ45" LOC = "M22";

    - # NET "DDR2_DQ46" LOC = "N25";

    - # NET "DDR2_DQ47" LOC = "N24";

    - # NET "DDR2_DQ48" LOC = "N20";

    - # NET "DDR2_DQ49" LOC = "P25";

    - # NET "DDR2_DQ5" LOC = "D18";

    - # NET "DDR2_DQ50" LOC = "P24";

    - # NET "DDR2_DQ51" LOC = "P23";

    - # NET "DDR2_DQ52" LOC = "R24";

    - # NET "DDR2_DQ53" LOC = "R23";

    - # NET "DDR2_DQ54" LOC = "T24";

    - # NET "DDR2_DQ55" LOC = "T23";

    - # NET "DDR2_DQ56" LOC = "U26";

    - # NET "DDR2_DQ57" LOC = "U23";

    - # NET "DDR2_DQ58" LOC = "V23";

    - # NET "DDR2_DQ59" LOC = "U25";

    - # NET "DDR2_DQ6" LOC = "A24";

    - # NET "DDR2_DQ60" LOC = "T21";

    - # NET "DDR2_DQ61" LOC = "T20";

    - # NET "DDR2_DQ62" LOC = "U20";

    - # NET "DDR2_DQ63" LOC = "T19";

    - # NET "DDR2_DQ64" LOC = "V22";

    - # NET "DDR2_DQ65" LOC = "W21";

    - # NET "DDR2_DQ66" LOC = "W22";

    - # NET "DDR2_DQ67" LOC = "W23";

    - # NET "DDR2_DQ68" LOC = "Y25";

    - # NET "DDR2_DQ69" LOC = "Y26";

    - # NET "DDR2_DQ7" LOC = "A23";

    - # NET "DDR2_DQ70" LOC = "AB24";

    - # NET "DDR2_DQ71" LOC = "AB25";

    - # NET "DDR2_DQ8" LOC = "B23";

    - # NET "DDR2_DQ9" LOC = "F18";

    - # NET "DDR2_DQS_N0" LOC = "B21";

    - # NET "DDR2_DQS_N1" LOC = "G17";

    - # NET "DDR2_DQS_N2" LOC = "A19";

    - # NET "DDR2_DQS_N3" LOC = "C24";

    - # NET "DDR2_DQS_N4" LOC = "C23";

    - # NET "DDR2_DQS_N5" LOC = "D25";

    - # NET "DDR2_DQS_N6" LOC = "E26";

    - # NET "DDR2_DQS_N7" LOC = "H25";

    - # NET "DDR2_DQS_N8" LOC = "K20";

    - # NET "DDR2_DQS0" LOC = "C21";

    - # NET "DDR2_DQS1" LOC = "G18";

    - # NET "DDR2_DQS2" LOC = "A20";

    - # NET "DDR2_DQS3" LOC = "D24";

    - # NET "DDR2_DQS4" LOC = "D23";

    - # NET "DDR2_DQS5" LOC = "D26";

    - # NET "DDR2_DQS6" LOC = "F26";

    - # NET "DDR2_DQS7" LOC = "H26";

    - # NET "DDR2_DQS8" LOC = "L19";

    - # NET "DDR2_ODT" LOC = "AF19";

    - # NET "DDR2_RAS_N" LOC = "AD23";

    - # NET "DDR2_WE_N" LOC = "AC24";

    - # NET "READ_EN_I0" LOC = "E17";

    - # NET "READ_EN_O0" LOC = "F17";

    - # NET "READ_EN_O1" LOC = "J22";

    - # NET "READ_EN_I1" LOC = "J23";

    - # NET "READ_EN_I2" LOC = "W25";

    - # NET "READ_EN_O2" LOC = "W26";

    - #

  - CPLD U11

    - CPLD Pin Map

    - CPLD Pin Description

| SIGNAL NAME | Type | LOGIC | DESCRIPTION |
| --- | --- | --- | --- |
| PS_IOG[4:1] | I | 1=On, 0=Off | Status Bits PS_IOG[1] = CH12’s +ve Rail Supply (+28V to +57V) PS_IOG[2] = CH12’s -ve Rail Supply (-28V to -57V) PS_IOG[3] = CH34’s +ve Supply (+28V to +57V) PS_IOG[4] = CH1’s -ve Supply (-28V to -57V) Note: These pins has to be pulled up Inside CPLD |
| CHx_ON | OUT | 1=Close, 0=Open | Ch1-Ch4 Output Relays Control: |
| CHx_KELVIN_HI | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to select resistor for Testing Kelvin Connection for High Side. Works I conjunction with Channel Outputs |
| CHx_KELVIN_LO | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to select resistor for Testing Kelvin Connection for Low Side. Works I conjunction with Channel Outputs |
| CHx_INT | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to connect Force to Feedback, thereby bypassing remote sense. |
| CHx_ADC_RNG_1V_N | OUT | 1=Open, 0=Close | Ch1-Ch4 Switch to select ADC 1V Range. CHx_VMU_10V_SEL has to be 1. |
| CHx_ ADC_RNG _2V_N | OUT | 1=Open, 0=Close | Ch1-Ch4 Switch to select ADC 2V Range. CHx_VMU_10V_SEL has to be 1. |
| CHx_ ADC_RNG _5V_N | OUT | 1=Open, 0=Close | Ch1-Ch4 Switch to select ADC 5V Range. CHx_VMU_10V_SEL has to be 1. |
| CHx_ ADC_RNG _10V_N | OUT | 1=Open, 0=Close | Ch1-Ch4 Switch to select ADC 10V Range. CHx_VMU_10V_SEL has to be 1. |
| CHx_VMU_DUT_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to connect DUT to ADC |
| CHx_VMU_CAL_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to connect Internal CAL tree to ADC |
| CHx_VMU_VI_E_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to connect channels’ voltage measure circuits to ADC |
| CHx_VMU_VI_I_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to connect channels’ current measure circuits to ADC |
| CHx_VMU_KHI_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to connect Kelvin High side test circuit to ADC |
| CHx_VMU_KLO_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to connect Kelvin Low side test circuit to ADC |
| CHx_VMU_10V_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to select the 1:1 attenuator for ADC. This is the 10V Range |
| CHx_VMU_20V_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to select the 2:1 attenuator for ADC. This is the 20V Range |
| CHx_VMU_50V_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to select the 5:1 attenuator for ADC. This is the 50V Range |
| CHx_VMU_100V_SEL | OUT | 1=Close, 0=Open | Ch1-Ch4 Relays to select the 10:1 attenuator for ADC. This is the 100V Range |
| CHx_ADC_PD | OUT | 1=On, 0=Off | Power Down Mode for ADC |
| CPLD_SCLK | I | POS | Serial Clock from FPGA |
| CPLD_SDATAI | I | POS | Serial Data I from FPGA |
| CPLD_STRB | I | POS | Strobe for transferring Register to IO pin |
| CPLD_SDATAOUT | OUT | POS | Serial Data OUT to FPGA for read back |

    - U11 UCF

NET  "CH1_ADC_PD"   LOC = "K1" ;

NET  "CH1_ADC_RNG_10V_N"   LOC = "H4" ;

NET  "CH1_ADC_RNG_1V_N"   LOC = "J4" ;

NET  "CH1_ADC_RNG_2V_N"   LOC = "J3" ;

NET  "CH1_ADC_RNG_5V_N"   LOC = "J1" ;

NET  "CH1_INT"   LOC = "G13" ;

NET  "CH1_KELVIN_HI"   LOC = "J12" ;

NET  "CH1_KELVIN_LO"   LOC = "J13" ;

NET  "CH1_ON"   LOC = "H12" ;

NET  "CH1_VMU_100V_SEL"   LOC = "H3" ;

NET  "CH1_VMU_10V_SEL"   LOC = "N3" ;

NET  "CH1_VMU_20V_SEL"   LOC = "M3" ;

NET  "CH1_VMU_50V_SEL"   LOC = "H1" ;

NET  "CH1_VMU_CAL_SEL"   LOC = "N4" ;

NET  "CH1_VMU_DUT_SEL"   LOC = "M4" ;

NET  "CH1_VMU_KHI_SEL"   LOC = "L4" ;

NET  "CH1_VMU_KLO_SEL"   LOC = "K4" ;

NET  "CH1_VMU_VI_E_SEL"   LOC = "L5" ;

NET  "CH1_VMU_VI_I_SEL"   LOC = "K5" ;

NET  "CH2_ADC_PD"   LOC = "M12" ;

NET  "CH2_ADC_RNG_10V_N"   LOC = "L10" ;

NET  "CH2_ADC_RNG_1V_N"   LOC = "L11" ;

NET  "CH2_ADC_RNG_2V_N"   LOC = "M11" ;

NET  "CH2_ADC_RNG_5V_N"   LOC = "N11" ;

NET  "CH2_INT"   LOC = "H13" ;

NET  "CH2_KELVIN_HI"   LOC = "J10" ;

NET  "CH2_KELVIN_LO"   LOC = "J11" ;

NET  "CH2_ON"   LOC = "H11" ;

NET  "CH2_VMU_100V_SEL"   LOC = "M10" ;

NET  "CH2_VMU_10V_SEL"   LOC = "L9" ;

NET  "CH2_VMU_20V_SEL"   LOC = "K9" ;

NET  "CH2_VMU_50V_SEL"   LOC = "N10" ;

NET  "CH2_VMU_CAL_SEL"   LOC = "L1" ;

NET  "CH2_VMU_DUT_SEL"   LOC = "M2" ;

NET  "CH2_VMU_KHI_SEL"   LOC = "N1" ;

NET  "CH2_VMU_KLO_SEL"   LOC = "N9" ;

NET  "CH2_VMU_VI_E_SEL"   LOC = "K2" ;

NET  "CH2_VMU_VI_I_SEL"   LOC = "K3" ;

NET  "CH3_ADC_PD"   LOC = "D1" ;

NET  "CH3_ADC_RNG_10V_N"   LOC = "A5" ;

NET  "CH3_ADC_RNG_1V_N"   LOC = "B1" ;

NET  "CH3_ADC_RNG_2V_N"   LOC = "C6" ;

NET  "CH3_ADC_RNG_5V_N"   LOC = "D6" ;

NET  "CH3_INT"   LOC = "B10" ;

NET  "CH3_KELVIN_HI"   LOC = "A9" ;

NET  "CH3_KELVIN_LO"   LOC = "B9" ;

NET  "CH3_ON"   LOC = "C9" ;

NET  "CH3_VMU_100V_SEL"   LOC = "B5" ;

NET  "CH3_VMU_10V_SEL"   LOC = "B4" ;

NET  "CH3_VMU_20V_SEL"   LOC = "A4" ;

NET  "CH3_VMU_50V_SEL"   LOC = "D5" ;

NET  "CH3_VMU_CAL_SEL"   LOC = "C3" ;

NET  "CH3_VMU_DUT_SEL"   LOC = "B3" ;

NET  "CH3_VMU_KHI_SEL"   LOC = "A3" ;

NET  "CH3_VMU_KLO_SEL"   LOC = "C4" ;

NET  "CH3_VMU_VI_E_SEL"   LOC = "A1" ;

NET  "CH3_VMU_VI_I_SEL"   LOC = "A2" ;

NET  "CH4_ADC_PD"   LOC = "M5" ;

NET  "CH4_ADC_RNG_10V_N"   LOC = "N7" ;

NET  "CH4_ADC_RNG_1V_N"   LOC = "M6" ;

NET  "CH4_ADC_RNG_2V_N"   LOC = "N6" ;

NET  "CH4_ADC_RNG_5V_N"   LOC = "M7" ;

NET  "CH4_INT"   LOC = "C10" ;

NET  "CH4_KELVIN_HI"   LOC = "C8" ;

NET  "CH4_KELVIN_LO"   LOC = "D8" ;

NET  "CH4_ON"   LOC = "D9" ;

NET  "CH4_VMU_100V_SEL"   LOC = "G3" ;

NET  "CH4_VMU_10V_SEL"   LOC = "F2" ;

NET  "CH4_VMU_20V_SEL"   LOC = "G2" ;

NET  "CH4_VMU_50V_SEL"   LOC = "G1" ;

NET  "CH4_VMU_CAL_SEL"   LOC = "E2" ;

NET  "CH4_VMU_DUT_SEL"   LOC = "E1" ;

NET  "CH4_VMU_KHI_SEL"   LOC = "F4" ;

NET  "CH4_VMU_KLO_SEL"   LOC = "F3" ;

NET  "CH4_VMU_VI_E_SEL"   LOC = "E4" ;

NET  "CH4_VMU_VI_I_SEL"   LOC = "E3" ;

NET  "CPLD_SCLK"   LOC = "C7" ;

NET  "CPLD_SDATAI"   LOC = "D7" ;

NET  "CPLD_SDATAOUT"   LOC = "K12" ;

NET  "CPLD_STRB"   LOC = "K11" ;

NET  "PS_IOG[1] "   LOC = "B12" ;  # Need PullUp … Open Drain

NET  "PS_IOG[2]"   LOC = "D12" ;  # Need PullUp … Open Drain

NET  "PS_IOG[3]"   LOC = "D13" ;  # Need PullUp … Open Drain

NET  "PS_IOG[4]"   LOC = "E10" ;  # Need PullUp … Open Drain

#NET  "SPARE_E1"   LOC = "K10" ;

#NET  "SPARE_E2"   LOC = "M8" ;

#NET  "SPARE_E3"   LOC = "L8" ;

#NET  "SPARE_E4"   LOC = "K8" ;

#NET  "SPARE_E5"   LOC = "K13" ;

#NET  "SPARE_E6"   LOC = "B7" ;

#NET  "SPARE_E7"   LOC = "A7" ;

  - CPLD CPx_U1

    - CPLD PIN Map

    - CPLD PIN Description

| SIGNAL NAME | Type | Logic | DESCRIPTION |
| --- | --- | --- | --- |
| CHx_5A | OUT | 1=Close 0=Open | ChA,ChB (A=1st, B=2nd Channels) Switches to select 5A Range |
| CHx_1A | OUT | 1=Close 0=Open | ChA,ChB Switches to select 1A Range |
| CHx_100MA | OUT | 1=Close 0=Open | ChA,ChB Switches to select 100mA Range |
| CHx_10MA | OUT | 1=Close 0=Open | ChA,ChB Switches to select 10mA Range |
| CHx_10MA_100UA_NSEN | OUT | 1=Close 0=Open | ChA,ChB Switches to select negative sense path for the 10mA to 100uA ranges |
| CHx_10MA_10NA_PSEN | OUT | 1=Close 0=Open | ChA,ChB Switches to select positive sense path for the 10mA to 10nA ranges |
| CHx_1MA | OUT | 1=Close 0=Open | ChA,ChB Switches to select 1mA Range |
| CHx_100UA | OUT | 1=Close 0=Open | ChA,ChB Switches to select 100uA Range |
| CHx_10UA | OUT | 1=Close 0=Open | ChA,ChB Switches to select 10uA Range |
| CHx_1UA | OUT | 1=Close 0=Open | ChA,ChB Switches to select 1uA Range |
| CHx_100NA | OUT | 1=Close 0=Open | ChA,ChB Switches to select 100nA Range |
| CHx_5A_100UA | OUT | 1=Close 0=Open | ChA,ChB Switches to select output Range of 100uA to 5A |
| CHx_10UA_10NA | OUT | 1=Close 0=Open | ChA,ChB Switches to select output Range of 10nA to 10uA |
| CHx_COMP[1:3]_N | OUT | 1=Open 0=Close | ChA,ChB Switches to set loop compensations for Voltage Control. |
| CHx_VSW[0:1] | OUT | 0=5V 1=50V 2=1V 3=10V | ChA,ChB Switches to Select the Voltage Range (Gain) |
| VFOLLOW | OUT | POS | ChB, Sets Channel B to follow Channel A |
| CHx_ICOMP_N | OUT |  | ChA,B Sets Loop Compensation for Current |
| CHx_SEL_X_IMON_N | OUT | 1=IT 0=EXT | ChA,B Selects External output/Input to control current Feedback |
| CHx_LOCAL_IMON_N | OUT | 1=EXT 0=IT | ChA,B Sets Current Feedback to Internal operation |
| CHx_50_RANGE_N | OUT |  | ChA,B Sets Voltage Loop Amp to 50V Range Use I Conjunction with CHx_5_10_RANGE_N and CHx_SLEW_N |
| CHx_5_10_RANGE_N | OUT |  | ChA,B Sets Voltage Loop Amp to 5-10V Range Use I Conjunction with CHx_50_RANGE_N and CHx_SLEW_N |
| CHx_SLEW_N | OUT |  | ChA,B Selects slew rate of Loop Amp. Use I Conjunction with CHx_5_10_RANGE_N and CHx_50_RANGE_N |
| SPARE_Exx_yy | ? | ? | SPARE pIs. Exx are the spare vias for Channels 1,2. Eyy are the spare vias for Channels 3,4 |
| VDAC_P_CHx_N | OUT | 1=NEG 0=POS | ChA, ChB Selects Positive / Negative Reference |
| DAC_SDI | OUT | POS | Quad DAC Serial Data I |
| DAC_CLK | OUT | POS | Quad DAC Serial Clock |
| DAC_CS_N | OUT | NEG | Quad DAC Chip Select |
| DAC_RS_N | OUT | NEG | Quad DAC Resets |
| DAC_LDAC_N | OUT | NEG | Quad DAC Load from Registers to DAC |
| SCLK | I | POS | Serial Clock from FPGA |
| SDATAI | I | POS | Serial Data I from FPGA |
| STRB | I | POS | Strobe for transferring Register to IO pin |
| SDATAOUT | OUT | POS | Serial Data OUT to FPGA for read back |

    - CPx_U1 UCF

    - NET  "SDATA_I"   LOC = "A7";

    - NET  "SCLK"   LOC = "B7";

    - NET  "STRB1"   LOC = "C7";

    - NET  "STRB2"   LOC = "D7";

    - NET  "SDATA_OUT"   LOC = "N1";

    - NET  "VFOLLOW_2"   LOC = "B11";

    - NET  "CHB_LOCAL_IMON_N"   LOC = "C10";

    - NET  "CHB_SEL_X_IMON_N"   LOC = "D10";

    - NET  "CHB_ICOMP_N"   LOC = "A11";

    - NET  "CHA_LOCAL_IMON_N"   LOC = "E2";

    - NET  "CHA_SEL_X_IMON_N"   LOC = "E3";

    - NET  "CHA_ICOMP_N"   LOC = "E4";

    - NET  "CHB_SLEW_N"   LOC = "G13";

    - NET  "CHB_5_10_RANGE_N"   LOC = "C8";

    - NET  "CHB_50_RANGE_N"   LOC = "D8";

    - NET  "CHB_VSW0"   LOC = "A9";

    - NET  "CHB_VSW1"   LOC = "B9";

    - NET  "CHB_COMP1_N"   LOC = "C9";

    - NET  "CHB_COMP2_N"   LOC = "D9";

    - NET  "CHB_COMP3_N"   LOC = "B10";

    - NET  "CHA_SLEW_N"   LOC = "N7";

    - NET  "CHA_5_10_RANGE_N"   LOC = "G3";

    - NET  "CHA_50_RANGE_N"   LOC = "G1";

    - NET  "CHA_VSW0"   LOC = "G2";

    - NET  "CHA_VSW1"   LOC = "F2";

    - NET  "CHA_COMP1_N"   LOC = "F3";

    - NET  "CHA_COMP2_N"   LOC = "F4";

    - NET  "CHA_COMP3_N"   LOC = "E1";

    - NET  "CHB_10UA"   LOC = "F12";

    - NET  "CHB_100UA"   LOC = "F10";

    - NET  "CHB_1MA"   LOC = "E13";

    - NET  "CHB_10MA_10NA_PSEN"   LOC = "E12";

    - NET  "CHB_10MA_100UA_NSEN"   LOC = "E11";

    - NET  "CHB_10MA"   LOC = "E10";

    - NET  "CHB_100MA"   LOC = "D13";

    - NET  "CHB_1A"   LOC = "D12";

    - NET  "CHB_10UA_10NA"   LOC = "A13";

    - NET  "CHB_5A_100UA"   LOC = "G11";

    - NET  "CHB_100NA"   LOC = "G10";

    - NET  "CHB_1UA"   LOC = "F13";

    - NET  "CHB_5A"   LOC = "B12";

    - NET  "CHA_10UA"   LOC = "D5";

    - NET  "CHA_100UA"   LOC = "A4";

    - NET  "CHA_1MA"   LOC = "B4";

    - NET  "CHA_10MA_10NA_PSEN"   LOC = "C4";

    - NET  "CHA_10MA_100UA_NSEN"   LOC = "A3";

    - NET  "CHA_10MA"   LOC = "B3";

    - NET  "CHA_100MA"   LOC = "C3";

    - NET  "CHA_1A"   LOC = "A2";

    - NET  "CHA_10UA_10NA"   LOC = "C6";

    - NET  "CHA_5A_100UA"   LOC = "D6";

    - NET  "CHA_100NA"   LOC = "A5";

    - NET  "CHA_1UA"   LOC = "B5";

    - NET  "CHA_5A"   LOC = "A1";

    - NET  "SPARE_E25_35"   LOC = "J12";

    - NET  "SPARE_E26_36"   LOC = "J13";

    - NET  "SPARE_E27_37"   LOC = "H11";

    - NET  "SPARE_E28_38"   LOC = "H12";

    - NET  "SPARE_E29_39"   LOC = "H13";

    - NET  "SPARE_E20_30"   LOC = "L5";

    - NET  "SPARE_E21_31"   LOC = "M5";

    - NET  "SPARE_E22_32"   LOC = "M6";

    - NET  "SPARE_E23_33"   LOC = "N6";

    - NET  "SPARE_E24_34"   LOC = "M7";

    - NET  "SPARE_E19_44"   LOC = "K2";

    - NET  "SPARE_E18_43"   LOC = "K3";

    - NET  "SPARE_E17_42"   LOC = "L1";

    - NET  "SPARE_E16_41"   LOC = "M2";

    - NET  "SPARE_E14_49"   LOC = "L10";

    - NET  "SPARE_E13_48"   LOC = "N11";

    - NET  "SPARE_E12_47"   LOC = "M11";

    - NET  "SPARE_E11_46"   LOC = "L11";

    - NET  "SPARE_E10_45"   LOC = "M12";

    - NET  "DAC_LDAC_N"   LOC = "H4";

    - NET  "DAC_RS_N"   LOC = "H3";

    - NET  "DAC_CS_N"   LOC = "H1";

    - NET  "DAC_CLK"   LOC = "M3";

    - NET  "DAC_SDI"   LOC = "N3";

    - NET  "VDAC_P_CHB_N"   LOC = "L4";

    - NET  "VDAC_P_CHA_N"   LOC = "N4";